Micro Project 3

Line Follower Robot

CYBERNETICS ROBO ACADEMY, SYLHET.

This is a line follower robot that can move following 3 cm black line on a white surface.

REQUIRED COMPONENTS:

| Components | Quantity | Photo |
| --- | --- | --- |
| 1. Arduino UNO | 1 |  |
| 2. Jumper Wire | 20 |  |
| 3. L298N Motor Driver | 1 |  |
| 4. Motor And Wheel | 2 |  |
| 5. Ball Causter | 1 |  |
| 6. Battery and Case | 1 set |  |
| 7. TCRT-5000 | 6 |  |
| 8. 220 ohm Resistor | 6 |  |
| 9. 20K Resistor | 6 |  |
| 10. Angel Male Header | 8 |  |
| 11. Vero board | 1 |  |
| 12. Chassis |  |  |

Step 1

Take the chassis and assemble it following the instruction in this video

https://www.youtube.com/watch?v=3a-bE1VlaU8

Step 2

Now put the Arduino, Motor Driver and battery case on the chassis like this

Step 3

Now take the Vero board, TCRT-5000, resistors, male angle header and solder them on the Vero board following the circuit diagram

Connections of the circuit in words:

1. 5V >> IR LED Positive terminal and Photo transistors Collector

2. IR LED negative Tarminal >> 220 ohm Resistor’s one end

3. Photo Transistors Emitter >> 20 k ohm Resistor’s one end
4. Both resistor’s another end >> Ground

5. Analog output will be taken from the Emitter.

After the circuit is completed it will look like this:

Connect Male angle headers as shown in the picture to power up the circuit and to take away the outputs.

Step 4

Now assemble the IR array under the robot just lie the picture. Make sure that the height of the sensors are not more than 1 cm from the surface.

Step 5

Now using jumper wires, Set the following connections:

1. Switch:

- One end >> Batter Positive Terminal

- Second end >> 12V input of motor driver.

2. Motor Driver:

- Gnd >> Battery Ground

- 12 V>> Switch

- En A >> Arduino 9

- En B >> Arduino 10

- IN 1 >> Arduino 5

- IN 2 >> Arduino 4

- IN 3 >> Arduino 3

- IN 4 >> Arduino 2

- OUT A & B >> Left Motor

- OUT C & D >> Right Motor

3. Sensor Array :

- Positive >> Motor driver 5 V

- Negative >> Bttery Ground

- Sensor 1 output (Left most motor) >> Ardudino A0

- Sensor 2 output >> Ardudino A1

- Sensor 3 output >> Ardudino A2

- Sensor 4 output >>  Ardudino A3

- Sensor 5 output >> Ardudino A4

- Sensor 6 output (Right most sensor) >> Ardudino A5

4. Arduino:

- Vin >> Motor driver 5V output

- Gnd >> Battery Ground

Step 6

Finally it’s time to run the code:

//////////////////////

//// CONNECTIONS  ////

//////////////////////

//    1. Switch:

//    •  One end >> Batter Positive Terminal

//    • Second end >> 12V input of motor driver.

//    2. Motor Driver:

//    • Gnd >> Battery Ground

//    • 12 V>> Switch

//    • En A >> Arduino 9

//    • En B >> Arduino 10

//    • IN 1 >> Arduino 5

//    • IN 2 >> Arduino 4

//    • IN 3 >> Arduino 3

//    • IN 4 >> Arduino 2

//    • OUT A & B >> Left Motor

//    • OUT C & D >> Right Motor

//    3. Sensor Array :

//    • Positive >> Motor driver 5 V

//    • Negative >> Bttery Ground

//    • Sensor 1 output (Left most motor) >> Ardudino A0

//    • Sensor 2 output >> Ardudino A1

//    • Sensor 3 output >> Ardudino A2

//    • Sensor 4 output >>  Ardudino A3

//    • Sensor 5 output >> Ardudino A4

//    • Sensor 6 output (Right most sensor) >> Ardudino A5

//    4. Arduino:

//    • Vin >> Motor driver 5V output

//    • Gnd >> Battery Ground

int sensor[]={A0,A1,A2,A3,A4,A5};//left to right that means sensor[0] indicates the left one and sensor[5] is the right one

int sv[]={0,0,0,0,0,0};// sensor value

//int tv[]={490+50,620+50,560+50,490+50,740+50,680+50};//threshold value

int tv[]={850,850,850,850,850,850};

int enL=9, enR=10; //enable of left motor and right motor

int motorPin[]={2,3,4,5,9,10};// last two is enable pin anr 1st four is control pin

int led[]={12,11,8,7,6};

int i;

int rs=150, ls=150; // enable value of right and left motor for going forward

void setup() {

Serial.begin(9600);

for(i=2;i<=12;i++){

//pinMode(motorPin[i],OUTPUT);

pinMode(i,OUTPUT);

}

}

void loop(){

sense(0); sense(5);

if (sv[0]<tv[0] || sv[5]<tv[5]){

hbreak();

follow(-100,-100);

for(;;){

sense(0);sense(5);

if(sv[0]<tv[0] || sv[5]<tv[5]){

Stop(10);

break;

}

}

sense(0); sense(5);

if(sv[0]<tv[0] && sv[5]<tv[5]){

follow(rs,ls);

for(;;){

sense(0);sense(5);

if(sv[0]>tv[0] && sv[5]>tv[5]){

Stop(100);

break;

}

}

}

else if(sv[0]<tv[0] && sv[5]>tv[5]){

int ang=scanangle(0);

if (ang==1){

follow(rs,ls);

for(;;){

sense(0);sense(1);

if(sv[0]>tv[0] && sv[1]>tv[1]){

Stop(100);

break;

}

}

follow(rs,-20);

for(;;){

sense(0);

if(sv[0]<tv[0]){

Stop(100);

break;

}

}

follow(20,-ls);

for(;;){

sense(2);

if(sv[2]<tv[2]){

Stop(100);

break;

}

}

}

else{

follow(rs,-ls);

for(;;){

sense(2);

if(sv[2]<tv[2])

break;

}

ledoff();

}

}

else if(sv[0]>tv[0] && sv[5]<tv[5]){

int ang=scanangle(5);

if (ang==1){

follow(rs,ls);

for(;;){

sense(4);sense(5);

if(sv[4]>tv[4] && sv[5]>tv[5]){

Stop(100);

break;

}

}

follow(-20,ls);

for(;;){

sense(5);

if(sv[5]<tv[5]){

Stop(100);

break;

}

}

follow(-rs,20);

for(;;){

sense(3);

if(sv[3]<tv[3]){

Stop(100);

break;

}

}

}

else{

ledF(0);

follow(-rs,ls);

for(;;){

sense(3);

if(sv[3]<tv[3]){

Stop(10);

break;

}

}

}

}

}

else{

sense(1);sense(4);

if (sv[1]<tv[1] || sv[4]<tv[4]){

if(sv[1]>sv[4]){

follow(-20,ls);

}

else if(sv[1]<sv[4]){

follow(rs,-20);

}

}

else{

sense(2);sense(3);

if (sv[2]>sv[3]+10){

follow(rs-60,ls+105);  //delay(500);

}

else if (sv[2]+10<sv[3]){

follow(rs+100,ls-50);

}

else{

follow(rs,ls);

}

}

}

}

void testMotors(){

for(;;){forward(255,255);}

for(int i=255;i>=-255;i--){

follow(i,i);

delay(10);

}

for(int i=-255;i<=255;i++){

follow(i,i);

delay(10);

}

}

void follow(int r, int l){

if (r>=0){

rwf(r);

}

else{

r=-r;

rwb(r);

}

if (l>=0){

lwf(l);

}

else{

l=-l;

lwb(l);

}

}

void backward(int bw){

rwb(bw);

lwb(bw);

}

void left(int lr, int ll){

rwf(lr);

lwb(ll);

}

void right(int rr, int rl){

rwb(rr);

lwf(rl);

}

void Stop(int s){

//follow(-rs,-ls);

//delay(10);

for(i=0;i<=5;i++)

digitalWrite(motorPin[i],0);

delay(s);

}

void hbreak(){

follow(-rs,-ls);

delay(70);

Stop(10);

}

void lwb(int a){ // right wheel forward

analogWrite(enL,a);

digitalWrite(motorPin[2],1);

digitalWrite(motorPin[3],0);

}

void lwf(int b){ // right wheel bacward

analogWrite(enL,b);

digitalWrite(motorPin[2],0);

digitalWrite(motorPin[3],1);

}

void rwf(int c){ // left wheel bacward

analogWrite(enR,c);

digitalWrite(motorPin[0],1);

digitalWrite(motorPin[1],0);

}

void rwb(int d){ // left wheel forwrd

analogWrite(enR,d);

digitalWrite(motorPin[0],0);

digitalWrite(motorPin[1],1);

}

void forward(int fr, int fl){

rwf(fr);

lwf(fl);

}

void checksensors(){

for(int i=0;i<6;i++){

sv[i]=analogRead(sensor[i]);

Serial.print(sv[i]);

Serial.print("    ");

}

Serial.println("");

}

int sense(int x){

sv[x]=analogRead(sensor[x]);

}

int scanangle(int x){

int val=0;

for(int i=0;i<6;i++){

if (i==x)

continue;

sv[i]=analogRead(sensor[i]);

if (sv[i]<tv[i]){

val=1;

break;

}

}

if (val==0){

sv[x]=analogRead(sensor[x]);

if (sv[x]>tv[x]){

val=1;

}

}

return val;

}

Step 7

That’s all. The robot is ready to follow over the line.